SCHOOL OF COMPUTER SCIENCE Assessment Task 1: Group Assignment – 30% Module Code; ITS67304. Module Name; Wireless Networks and Security. Assessment; Assessment 3 (30%). Student Name; Kriteishini A/P R G Elangovan. Student ID; 0366545. Student Name; Arvin Raj A/L Neethiahnanthan. Student ID; 0366957. Student Name; Lau Hui Henk. Student ID; 0365791. Semester/Year; September 2025.

Marking Rubrics:

| Criteria | 90-100% (Excellent) | 70-89% (Good) | 50-69% (Needs Improvement) | 0-49% (Poor) | Weight | Your Score |
| --- | --- | --- | --- | --- | --- | --- |
| Abstract | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 4 |  |
| Video and Presentation | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 10 |  |
| Introduction to Scenario | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 2 |  |
| Background | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 1 |  |
| Identify Key Security Issues | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 1 |  |
| Analyze the Problem | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 3 |  |
| Solve the Problem | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 4 |  |
| Literature Review | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 2 |  |
| Discussion | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 1 |  |
| Reflection | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 1 |  |
| References | Comprehensive and complete in all aspect | Good content and cover most aspect | Average content and cover some aspect | Poor content and incomplete | 1 |  |
|  |  |  |  | Total | 30 |  |

Table of Contents

1.0 Abstract	6

2.0 Introduction	7

3.0 Real-World Scenario	8

3.1 Student Network	8

3.2 Staff Network	8

3.3 IoT Sensor Network	8

3.4 Rationale for Scenario Selection	8

3.4.1 Complexity of the Attack Surface	8

3.4.2 Integration of Legacy Technologies	9

3.4.3 Application of Defense in Depth	9

4.0 Case Background	10

4.1 Introduction to the Environment	10

4.2 Architecture Overview	10

4.3 Description of Network Zones and Threat Scenarios	11

4.3.1 Student Zone (VLAN 10)	11

4.3.2 Staff Network (VLAN 20)	11

4.3.3 IoT Sensor Network (VLAN 30)	11

4.4 Purpose of the Simulation	12

5.0 Identification of Key Security Issues	13

5.1 Student Network (VLAN 10)	13

5.2 Staff Network (VLAN 20)	13

5.3 IoT Sensor Network (VLAN 30)	14

6.0 Analysis and Solution of Problems	15

6.1 Student Network (VLAN 10)	15

6.2 Staff Network (VLAN 20)	15

6.3 IoT Sensor Network (VLAN 30)	16

7.0 Supporting Material	17

7.1 Student Zone (VLAN 10): Fake Wi-Fi Signals	17

7.2 Staff Network (VLAN 20): Man-in-the-Middle Attacks	17

7.3 IoT Sensor Network (VLAN 30): Sending Passwords in PlainText	17

8.0 Discussion and Reflection	19

8.1 Discussion	19

8.2 Reflection on Campus Network Security	19

8.2.1 Security is About People, Not Just Technology	19

8.2.2 Old Technology is a Major Risk	19

8.2.3 Trust Needs to Be Verified	20

9.0 Conclusion	21

10.0 References	22

# 1.0 Abstract

This report examines the security challenges and mitigation strategies associated with a Smart Campus Wireless Security Architecture. The study focuses on three critical network segments: the Student Network, the Staff Network, and the Internet of Things (IoT) Sensor Network. Each segment presents distinct security vulnerabilities due to differences in user behaviour, device capabilities, and access requirements. The student network is exposed to rogue access point attacks resulting from unmanaged personal devices and unverified wireless connections. The staff network faces increased risk of man-in-the-middle attacks due to insufficient mutual authentication, while the IoT sensor network relies on legacy protocols such as Telnet that transmit sensitive data in plaintext.

To address these vulnerabilities, the report adopts a defense in depth approach that integrates multiple layers of security controls. The proposed solutions include network segmentation using virtual local area networks, client isolation, the application of encryption mechanisms, and the enforcement of access control lists to restrict inter-network communication. These measures are designed to minimise the attack surface, contain potential threats, and protect sensitive institutional data.

Through the analysis of realistic attack scenarios and the implementation of targeted mitigation strategies, this report demonstrates how a Smart Campus can achieve a secure and resilient wireless network architecture. The findings highlight the importance of aligning security controls with user roles and device trust levels to ensure reliable connectivity while maintaining robust protection against evolving wireless security threats.

# 2.0 Introduction

The rapid adoption of Smart Campus technologies has significantly increased universities’ dependence on wireless networks to support academic delivery, administrative operations, and automated facility management. Wireless Local Area Networks now serve as the primary communication backbone for a wide range of devices, including student-owned laptops and smartphones, staff workstations, and Internet of Things sensors deployed across campus infrastructure. As a result, ensuring the security and reliability of campus wireless environments has become a critical concern.

Smart Campus networks present unique security challenges due to their heterogeneous nature. The widespread use of Bring Your Own Device models introduces unmanaged and potentially vulnerable endpoints, while legacy IoT devices often lack support for modern security protocols. At the same time, staff networks must protect sensitive institutional data from unauthorized access and interception. These factors collectively increase the attack surface and complicate the implementation of effective security controls.

This report investigates the security risks associated with a Smart Campus Wireless Security Architecture and evaluates mitigation strategies using a defense in depth approach. The analysis focuses on three key network segments: the Student Network, the Staff Network, and the IoT Sensor Network. By identifying vulnerabilities within each segment and proposing targeted security controls, the report aims to demonstrate how layered security mechanisms can enhance the overall resilience of campus wireless networks while maintaining accessibility and performance.

# 3.0 Real-World Scenario

The selected scenario for this project is “Smart Campus Wireless Security Architecture.” A Smart Campus represents a modern educational environment in which wireless connectivity is a fundamental operational requirement rather than an auxiliary service (Yang et al., 2023). Universities such as Taylor’s University depend on large-scale wireless LAN (WLAN) infrastructures capable of supporting thousands of users concurrently. These networks must sustain high bandwidth demands for e-learning systems and administrative platforms, while also accommodating a rapidly expanding ecosystem of Internet of Things (IoT) devices for campus automation and security (Min-Allah and Alayed, 2020; Al-Turjman, 2022).

This environment is particularly challenging from a security perspective because it must simultaneously serve three user groups with significantly different levels of trust, device diversity, and operational requirements.

## 3.1 Student Network

Students represent the largest and most unpredictable user population in the campus environment. They rely on a Bring Your Own Device model, in which thousands of personal devices such as laptops and smartphones connect to the network. Many of these devices lack updated patches, security configurations, or antivirus protection. This places the student network at high risk of malware propagation and unauthorized access attempts (Tu and Yuan, 2021).

## 3.2 Staff Network

Staff members require access to confidential institutional information such as academic records and financial data. Their work involves systems that demand strong confidentiality and integrity. The staff network must therefore be designed with strict access restrictions to ensure that unauthorized users cannot intercept sensitive communication or reach internal servers (Stallings, 2023).

## 3.3 IoT Sensor Network

The Smart Campus relies on an extensive IoT ecosystem, including automated lighting and networked CCTV cameras. These devices often operate on lightweight or outdated firmware that does not support contemporary encryption standards (El‐Hajj, 2024). Many still rely on unencrypted communication protocols such as Telnet. This creates a weak link in the wider security architecture because attackers can exploit these devices as entry points into other parts of the network (NIST, 2021).

## 3.4 Rationale for Scenario Selection

This scenario was selected for the following reasons.

### 3.4.1 Complexity of the Attack Surface

The campus environment is inherently open and accessible to students, staff, and visitors. This increases the number of potential attack vectors. Unlike corporate networks, threats may originate from within the physical premises, making the environment significantly more difficult to secure (Ulven & Wangen, 2021).

### 3.4.2 Integration of Legacy Technologies

A Smart Campus must integrate modern IT systems with older operational technologies. Legacy IoT devices such as CCTV cameras often rely on insecure protocols, notably Telnet, that expose credentials and configuration data in plaintext. This integration challenge presents realistic and relevant risks for simulation and analysis (Hassija et al., 2021).

### 3.4.3 Application of Defense in Depth

This scenario provides the opportunity to demonstrate multiple layers of security. These include network segmentation using VLANs, strong wireless authentication mechanisms such as WPA2 and WPA3, traffic isolation, and monitoring tools aimed at mitigating threats like eavesdropping, rogue access points, and man in the middle attacks (Abedi et al., 2023).

# 4.0 Case Background

## 4.1 Introduction to the Environment

The transition from a traditional university infrastructure to a Smart Campus ecosystem represents a significant shift in how digital services are delivered and consumed. A Smart Campus integrates pervasive wireless connectivity, cloud-based educational platforms, and automated building management systems to create a hyper-connected learning environment (Polin et al., 2023). In this context, wireless networking becomes a critical operational backbone rather than merely a supplementary convenience. Reliable WLAN coverage is essential to support e-learning platforms, administrative workflows, and the increasing number of wireless Internet of Things (IoT) devices deployed across campus facilities.

The importance of wireless infrastructure continues to grow as students and staff rely heavily on mobile learning, flexible workspaces, and cloud-based services. Fixed cabling can no longer meet these demands due to the high density of roaming users and the requirement for seamless connectivity (Finch et al., 2021). Additionally, the Smart Campus model must serve three distinct user groups; students, staff, and IoT devices, each of which presents unique security requirements and operational behaviours. These differences contribute to a complex and heterogeneous wireless environment that requires rigorous security design and monitoring.

## 4.2 Architecture Overview

Figure 4.2 Network Topology of the Smart Campus, illustrating the segmentation of the Student, Staff, IoT, and Server Networks with respective VLANs and key devices.

To model the operational environment of a Smart Campus, a wireless network topology was designed and simulated using Cisco Packet Tracer. The logical architecture is built around a centralized backbone router that manages traffic flow between multiple network segments. To reduce the attack surface and enforce strict isolation, the campus network is segmented into Virtual Local Area Networks (VLANs), each serving a specific operational domain:

- VLAN 10: Student Network (Public)

- VLAN 20: Staff Network (Private)

- VLAN 30: IoT Network (Automation)

- VLAN 99: Server Farm (Management)

Dedicated access points provide wireless services for the student, staff, and IoT zones, while a centralized server farm hosts key network services such as the RADIUS authentication server and the Wazuh Security Information and Event Management (SIEM) system. A visual representation of this topology is provided in Figure X, illustrating the segmentation and communication routes between the core network components.

## 4.3 Description of Network Zones and Threat Scenarios

### 4.3.1 Student Zone (VLAN 10)

The student zone is a high-volume, public-facing wireless segment that supports personal devices such as laptops and smartphones (e.g., ST_Laptop1, ST_Laptop2). As students frequently prioritise convenience over security, they commonly connect to any available Wi-Fi signal that appears legitimate (Palamà et al., 2023) (Pósa & Großklags, 2022). This behaviour exposes them to attacks such as rogue access points, where an attacker broadcasts a cloned SSID to deceive users. Such an attack allows the adversary to intercept traffic, harvest credentials, or redirect users to malicious destinations (O'Reilly & Balaguer, 2020). This segment therefore represents one of the most vulnerable areas within the campus environment due to user behaviour and device diversity.

### 4.3.2 Staff Network (VLAN 20)

The staff network supports faculty and administrative personnel who require secure access to sensitive resources, including grading systems and internal administrative servers. Devices in this network segment (e.g., SF_Laptop1) handle confidential information, making the zone a high-value target for attackers. The initial configuration lacks robust mutual authentication mechanisms between client devices and the access points, creating opportunities for adversaries to launch man-in-the-middle (MitM) attacks. In such scenarios, attackers can intercept or manipulate communication streams while appearing to be part of the legitimate network (Stallings, 2023). This vulnerability underscores the necessity of strong authentication and encryption policies for staff wireless access.

### 4.3.3 IoT Sensor Network (VLAN 30)

The IoT network consists of campus physical security devices, including CCTV cameras (e.g., IoT_Sensor1) used for real-time facility monitoring. Many of these devices rely on legacy communication protocols such as Telnet over port 23, which transmits all data including login credentials in plaintext (Gentile et al., 2024). The absence of encryption exposes the network to packet sniffing and credential theft, enabling attackers to gain unauthorized control of surveillance devices (NIST, 2021). The limited security capabilities and outdated firmware common to many IoT devices increase the overall attack surface and make this segment particularly vulnerable to intrusion.

## 4.4 Purpose of the Simulation

The primary objective of the simulation is to evaluate and stress-test the security posture of the Smart Campus wireless architecture under realistic threat conditions. The project recreates attacks against each network segment to demonstrate how theoretical vulnerabilities materialize into practical exploits. The scenarios provide insight into the weaknesses present in an “as-is” configuration prior to the implementation of hardening mechanisms. Furthermore, the simulation facilitates the testing of targeted defence strategies such as replacing Telnet with SSH for IoT device management which are discussed in the solution section of this report. Through this approach, the project offers a comprehensive assessment of the campus wireless environment and identifies practical measures to enhance security resilience.

# 5.0 Identification of Key Security Issues

The Smart Campus Wireless Security Architecture is structured around three primary wireless segments: the Student Network (VLAN 10), the Staff Network (VLAN 20), and the IoT Sensor Network (VLAN 30). These segments support users with different device profiles, trust levels, and security expectations. As a result, each zone presents specific vulnerabilities that must be identified prior to implementing appropriate security controls. This section outlines the key security issues present within each network segment, providing a necessary foundation for the detailed analysis and mitigation strategies discussed in the following section.

## 5.1 Student Network (VLAN 10)

A major vulnerability within the student network is its susceptibility to rogue access point or “evil twin” attacks (Kara, 2024). Students commonly connect to wireless networks based on signal strength rather than AP legitimacy, making them particularly vulnerable when attackers clone the campus SSID to deceive users. Once connected, the attacker can harvest credentials or intercept unencrypted traffic (Palamà et al., 2023).

Another significant issue is the absence of a standardized security baseline for student-owned devices. Since, the campus operates under a BYOD model, these devices vary substantially in their operating systems, patch levels, antivirus configurations, and overall security posture (Ntwari et al., 2021). Such diversity introduces considerable risk, as compromised or poorly secured devices can propagate malware, exploit open services, or act as entry points for internal threats (Boeckl et al., 2021).

The BYOD network is further challenged by high levels of broadcast traffic and congestion. With hundreds or thousands of simultaneously connected devices, wireless contention and interference increase dramatically. Excessive ARP requests, DHCP queries, and general broadcast traffic degrade network performance and elevate the risk of unintentional denial-of-service conditions, resulting in unstable connectivity for all users (Alamleh et al., 2025).

## 5.2 Staff Network (VLAN 20)

One of the most critical issues in the staff VLAN is its exposure to man-in-the-middle (MitM) attacks. In the absence of robust mutual authentication mechanisms, attackers can conduct ARP spoofing or broadcast fraudulent access points to intercept sensitive communication, including academic records, administrative data, or login credentials (Morsy & Nashat, 2022).

A secondary vulnerability arises from weak access control enforcement. When staff authentication is limited to WPA2-PSK or other simplified mechanisms, unauthorized devices may be able to join the staff network (Palamà et al., 2023). This increases the risk of insider threats and allows attackers to gain proximity to confidential resources.

A third concern is the possibility of lateral movement toward high-value servers. Staff devices routinely interact with grading systems, database servers, and HR platforms. If ACLs or inter-VLAN restrictions are misconfigured, a compromised staff device could be exploited as a pivot point to access critical systems, resulting in data breaches or privilege escalation (Jin et al., 2023).

## 5.3 IoT Sensor Network (VLAN 30)

The IoT network contains multiple vulnerabilities rooted in the limitations of legacy IoT hardware. A major issue is the continued use of Telnet and other unencrypted management protocols. These protocols transmit login credentials and configuration data in plaintext, enabling attackers to capture sensitive information through packet sniffing and subsequently take control of the device (Pütz et al., 2023).

Another issue is the inability of many IoT sensors to support modern encryption standards such as WPA3 or certificate-based authentication. Due to firmware constraints and hardware limitations, these devices rely on weaker protection mechanisms, creating a structural weakness that attackers can exploit through wireless sniffing or unauthorized association attempts (Lakhani & Sachan, 2024).

Furthermore, many IoT devices lack adequate hardening. They often operate with outdated firmware, default credentials, and minimal security features (Coston et al., 2025). This makes them highly vulnerable to brute-force attacks, unauthorized remote access, and malware infections. If compromised, these devices can also serve as pivot points into other network segments (Shahid, 2021).

# 6.0 Analysis and Solution of Problems

The vulnerabilities identified within the Smart Campus architecture pose significant threats to the confidentiality, integrity, and availability of network resources across all three VLAN segments. These weaknesses arise from a combination of user behaviour, legacy technologies, and inadequate access control mechanisms (Ahmadi, 2025). This section provides a detailed analysis of each problem, discussing the underlying cause, the impacts associated with exploitation, and the specific mitigation strategies implemented or recommended. The objective is to propose practical and technically sound solutions that enhance the security posture of the student, staff, and IoT network zones.

## 6.1 Student Network (VLAN 10)

The student network has a major vulnerability due to rogue access point attacks. Student devices automatically reconnect to known SSIDs without verifying the legitimacy of the access point. Since WPA2-PSK does not authenticate the access point, attackers can broadcast a cloned SSID to trick users into connecting. Once connected, the attacker can intercept traffic, steal credentials, inject malware, or redirect users to phishing sites (Palamà et al., 2023). To mitigate this, the campus WLAN should use MAC filtering on the student access point to allow only registered devices to connect. The SSID should not be broadcast publicly to reduce visibility, and the WPA2 key should be changed every semester (Palamà et al., 2023). Enabling client isolation can prevent peer-to-peer attacks, and periodic security awareness programs can teach students how to connect safely (Lim et al., 2025).

Another issue in BYOD environments is that unmanaged personal devices may lack antivirus software, security patches, or proper configurations. These devices can spread malware, exploit open services, or generate DDoS traffic (Boeckl et al., 2021). To reduce this risk, VLAN isolation should be strictly enforced so that student devices cannot access sensitive network segments or communicate laterally. Access control lists (ACLs) on the core switch should block unnecessary inter-VLAN traffic. Rate limiting on access ports can prevent broadcast storms from compromised devices (Bhuse et al., 2024).

The third issue involves network congestion from the high volume of student devices generating ARP broadcasts, DHCP requests, and wireless communication. Excessive traffic reduces connection quality and increases the risk of denial-of-service conditions (Alamleh et al., 2025). Deploying multiple access points with load-balancing features, optimizing 5 GHz channel allocation to reduce interference, and configuring a centralized DHCP server with relay services can help address this issue (Ryan et al., 2025).

## 6.2 Staff Network (VLAN 20)

A significant vulnerability in the staff network is its exposure to man-in-the-middle attacks through ARP spoofing. Without ARP verification, attackers can forge ARP replies and redirect traffic to a malicious intermediary. This can lead to credential theft, session hijacking, and unauthorized access to sensitive data (Palamà et al., 2023). To address this, enable Dynamic ARP Inspection (DAI) to validate ARP packets against DHCP Snooping bindings. This ensures that only legitimate IP-MAC mappings are accepted. MAC filtering on the staff access point can limit access to authorized devices and reduce impersonation risks (Jadhav et al., 2023).

Weak authentication controls are another issue. If staff members use a shared WPA2-PSK, former employees or external attackers can obtain the password and infiltrate the network. This increases the risk of insider threats and unauthorized disclosure of confidential information. Transitioning to WPA2-Enterprise with RADIUS-based verification is recommended. In the meantime, device registration and MAC filtering can be used as interim solutions. Reducing the physical broadcast coverage of the staff SSID can limit its exposure outside of critical areas (Palamà et al., 2023).

The risk of lateral movement toward the server farm is another concern. A compromised staff laptop could try to access administrative systems within the 10.0.99.0/24 subnet (Jin et al., 2023). To mitigate this, enforce strict ACL policies that allow staff access only to necessary server ports and block all other unsolicited traffic. A router-on-a-stick architecture can ensure that all inter-VLAN traffic is routed through a controlled inspection point where security policies can be enforced (Álvarez et al., 2023).

## 6.3 IoT Sensor Network (VLAN 30)

The main vulnerability in the IoT VLAN is the use of Telnet for device management. Legacy cameras and sensors on TCP port 23 transmit login credentials and configuration data in cleartext, making them vulnerable to packet sniffing. Attackers can intercept usernames and passwords, take control of the devices, or move into other network segments (Pütz et al., 2023). To fix this, ACLs should restrict Telnet access to the IoT management server in the simulation. In real deployments, replace Telnet with secure protocols like SSH or TLS. Implementing IoT-specific firewall rules on the core switch can reduce the risk of lateral movement (Hwang & Kim, 2021).

Another issue is the lack of modern encryption support in legacy IoT firmware. This prevents the use of WPA3 or certificate-based authentication, increasing the risk of sniffing attacks and unauthorized device associations (Lakhani & Sachan, 2024). To mitigate this, IoT devices should be isolated within VLAN 30, and MAC filtering should ensure that only authorized sensors can join the network. Assigning static IP addresses to IoT devices can reduce DHCP-related attack vectors and improve traceability (Zaidi, 2024).

Many IoT sensors also use weak or default credentials, making them susceptible to brute-force attacks or unauthorized access (Shahid, 2021). To address this, enforce strong, unique passwords across all IoT endpoints, disable unused services, and apply regular firmware updates. Continuous monitoring through a security information and event management (SIEM) platform, like Wazuh, can provide real-time detection of unusual traffic patterns and unauthorized access attempts (Kabir et al., 2023).

The analysis demonstrates that each VLAN within the Smart Campus network exhibits distinct vulnerabilities stemming from user behaviour, misconfigurations, and inherent limitations of legacy technology. Implementing the proposed mitigation strategies including MAC filtering, VLAN segmentation, ACL enforcement, DHCP Snooping, DAI, and secure communication protocols significantly strengthens the overall security posture of the WLAN architecture. Collectively, these measures provide defence-in-depth against rogue access point attacks, man-in-the-middle threats, and IoT exploitation, thereby enhancing the resilience and reliability of the Smart Campus environment.

# 7.0 Supporting Material

This section reviews recent literature related to the security threats identified in the Smart Campus Wireless Security Architecture. The discussion focuses on the risks associated with mixed-trust wireless environments that support student-owned devices, staff systems handling sensitive data, and legacy IoT devices. The literature provides theoretical and practical support for the vulnerabilities demonstrated in the simulated campus network.

## 7.1 Student Zone (VLAN 10): Fake Wi-Fi Signals

The Student BYOD Network presents a high-risk environment due to the large number of unmanaged personal devices connecting to the wireless infrastructure. Students typically prioritise connectivity and performance over security awareness, which increases their susceptibility to Rogue Access Point attacks. In such attacks, adversaries deploy malicious access points that mimic legitimate campus SSIDs to deceive users into connecting.

Portnox (n.d.) explains that rogue access points are particularly effective in open wireless environments because users rarely verify the authenticity of access points before connecting. Once connected, attackers may intercept traffic, harvest credentials, or redirect users to malicious websites. Christie et al. (2025) further highlight that the diversity of operating systems, patch levels, and security configurations in BYOD environments makes it difficult for institutions to enforce a consistent security baseline, thereby expanding the internal attack surface. Minella (2022) emphasises that without proper wireless authentication and verification mechanisms, users can easily be tricked into connecting to malicious devices that appear legitimate.

## 7.2 Staff Network (VLAN 20): Man-in-the-Middle Attacks

The Staff Network supports access to sensitive institutional resources, including academic records, administrative systems, and internal servers. Due to the confidentiality of the data involved, this segment requires strong authentication and encryption mechanisms. However, inadequate mutual authentication between client devices and access points exposes the network to Man-in-the-Middle attacks.

Minella (2022) warns that enterprise wireless networks lacking robust encryption and certificate-based authentication are vulnerable to interception and session hijacking. In such scenarios, attackers can position themselves between staff devices and legitimate servers to capture credentials, steal session cookies, or manipulate transmitted data. Christie et al. (2025) further note that these attacks can lead to data tampering and information disclosure, posing serious risks to institutional integrity and trust.

## 7.3 IoT Sensor Network (VLAN 30): Sending Passwords in PlainText

The IoT Sensor Network hosts campus CCTV cameras that rely on legacy communication protocols such as Telnet operating on Port 23. Telnet transmits credentials and command data in plaintext, making it highly vulnerable to packet sniffing and credential theft.

Klein and Walcott (2021) strongly discourage the use of Telnet in modern networks, stating that any attacker with network access can easily capture sensitive information transmitted in cleartext. This vulnerability aligns with MITRE’s CWE-319 classification, which identifies the transmission of sensitive information without encryption as a critical weakness (MITRE, 2024). Additionally, OWASP (2024) lists insecure network services as a major threat in IoT environments. Baran (2025) further observes that many IoT devices operate on outdated firmware and lack built-in security mechanisms, making them attractive targets for attackers seeking initial access to larger networks.

# 8.0 Discussion and Reflection

## 8.1 Discussion

The analysis of the Smart Campus scenario confirms that a flat network topology is insufficient for a mixed-use environment. The proposed solution addresses the specific security voids identified in the literature through a Defense in Depth strategy.

- Securing the BYOD Environment: To counter Rogue Access Point threats in the Student Network (VLAN 10), technical controls must be robust. Our solution mitigates the risks identified by (Angella Christie et al., 2025) by implementing Client Isolation and Network Segmentation Isolating student devices in a dedicated VLAN (VLAN 10) prevents the lateral spread of malware from unmanaged personal devices to critical infrastructure.

- Mitigating IoT Legacy Risks: The simulation confirmed that legacy IoT devices (VLAN 30) using unencrypted Telnet are easy targets for eavesdropping. To address this, we implemented Access Control Lists (ACLs Strictly limiting Telnet traffic to originate only from the secure management server neutralizes the risk of an external attacker capturing credentials via packet sniffing. This aligns with industry best practices which suggest creating specific firewall rules to limit IoT access to only necessary service protocols.

- The Staff Network is a high-risk area because it handles sensitive data like student grades and admin passwords. The main issue identified is the lack of mutual authentication, which makes the network vulnerable to Man-in-the-Middle (MitM) attacks. This means a hacker can easily intercept the connection between a staff laptop and the server. (Jennifer Minella, 7 March 2022) warns that without strong validation and encryption, enterprise networks cannot protect confidential data from being stolen during transmission. (Angella Christie et al., 2025) further argue that this weakness allows attackers to tamper with information or steal login credentials, making it critical to upgrade the security protocols immediately.

## 8.2 Reflection on Campus Network Security

### 8.2.1 Security is About People, Not Just Technology

One key takeaway from analyzing the Student Zone is that, even with great hardware, human behavior remains a major vulnerability. Students often prioritize fast internet over security and do not consider the risks involved. This helped us understand why Rogue Access Points are so dangerous, as hackers exploit people’s desire for convenience. We realized that security isn't only about blocking ports; it's also about educating users on being cautious about what they connect to.

### 8.2.2 Old Technology is a Major Risk

Examining the IoT Sensor Network was an eye-opener. We were surprised that devices as critical as security cameras still used Telnet, a protocol that sends passwords in plain text. This highlighted how physical security measures like CCTV can create digital security gaps if the devices are outdated. We understood the importance of upgrading legacy systems and ensuring that sensitive devices never rely on unencrypted connections.

### 8.2.3 Trust Needs to Be Verified

The Staff Network scenario showed us that we cannot assume a connection is secure just because it’s within the building. The risk of Man-in-the-Middle (MitM) attacks emphasized that both devices and servers need to verify each other’s identity. Without proper verification, sensitive data such as grades and admin passwords are easily vulnerable to theft.

8.3 Conclusion

Overall, this project taught us that there is no “one-size-fits-all" solution for network security. Students need protection from their own carelessness; staff need strong encryption for their sensitive data and IoT devices need to be locked down because they can't protect themselves. I learned that a secure network requires looking at exactly who is using it and what they are doing, rather than just applying the same rules everywhere.

# 9.0 Conclusion

This project highlighted the importance of securing wireless networks in a Smart Campus environment. We focused on the vulnerabilities within the Student Network, Staff Network, and IoT Sensor Network, each presenting unique challenges. Our solutions, based on a Defense in Depth strategy, aimed to address these vulnerabilities by implementing robust security measures, such as network segmentation, client isolation, and encryption. Through this approach, we demonstrated that securing a Smart Campus requires a tailored strategy that considers the specific needs of different user groups. Improving security allows the Smart Campus network to better protect sensitive data and ensure a reliable, secure environment for all users.

# 10.0 References

Abedi, A., Lu, H., Chen, A., Liu, C., & Omid Abari. (2023). WiFi Physical Layer Stays Awake and Responds When it Should Not. IEEE Internet of Things Journal, 1–1. https://doi.org/10.1109/jiot.2023.3300788

Ahmadi, S. (2025). Distributed Identity for Zero Trust and Segmented Access Control: A Novel Approach to Securing Network Infrastructure. https://doi.org/10.20944/preprints202501.0917.v1

Alamleh, H., Estremera, L., Arnob, S. S., & AlQahtani, A. A. S. (2025). Advanced Persistent Threats and Wireless Local Area Network Security: An In-Depth Exploration of Attack Surfaces and Mitigation Techniques. Journal of Cybersecurity and Privacy, 5(2), 27. https://doi.org/10.3390/jcp5020027

Álvarez, D., Nuño, P., González, C. T., Bulnes, F. G., Granda, J. C., & García-Carrillo, D. (2023). Performance Analysis of Software-Defined Networks to Mitigate Private VLAN Attacks. Sensors, 23(4), 1747. https://doi.org/10.3390/s23041747

Antonio Francesco Gentile, Davide Macrì, Domenico Luca Carnì, Greco, E., & Lamonaca, F. (2024). A Performance Analysis of Security Protocols for Distributed Measurement Systems Based on Internet of Things with Constrained Hardware and Open Source Infrastructures. Sensors, 24(9), 2781–2781. https://doi.org/10.3390/s24092781

Aziz, M., Elmedany, W., & Sharif, M. S. (2023). Securing IoT devices against emerging security threats: Challenges and mitigation techniques. Journal of Cyber Security Technology, 7(4), 1–25. https://doi.org/10.1080/23742917.2023.2228053

Baran, E. (2025, March 26). Cyber Security Risks In Internet Of Things Devices. Blaze Information Security. https://www.blazeinfosec.com/post/cyber-security-risks-in-iot-devices/

Boeckl, K., Grayson, N., Howell, G., Lefkovitz, N., Ajmo, J., McGinnis, M., Sandlin, K., Slivina, O., Snyder, J., & Ward, P. (2022, November 29). Mobile Device Security: Bring Your Own Device (BYOD). Csrc.nist.gov. https://csrc.nist.gov/pubs/sp/1800/22/2pd

Christie, A., Hebrina, S., & Wijaya, G. E. (2025, December 9). Campus WiFi Network Security Analysis Using STRIDE Framework and Zero Trust Architecture Recommendations. ResearchGate; unknown. https://www.researchgate.net/publication/398477253_Campus_WiFi_Network_Security_Analysis_Using_STRIDE_Framework_and_Zero_Trust_Architecture_Recommendations

Coston, I., Plotnizky, E., & Nojoumian, M. (2025). Comprehensive Study of IoT Vulnerabilities and Countermeasures. Applied Sciences, 15(6), 3036. https://doi.org/10.3390/app15063036

CWE - CWE-319: Cleartext Transmission of Sensitive Information (4.3). (n.d.). Cwe.mitre.org. https://cwe.mitre.org/data/definitions/319.html

El-Hajj, M. (2024). Leveraging Digital Twins and Intrusion Detection Systems for Enhanced Security in IoT-Based Smart City Infrastructures. Electronics, 13(19), 3941. https://doi.org/10.3390/electronics13193941

Finch, K., Kala, S., & Sathya, V. (2021). The    Review    of    Mobile    Applications    and    Wireless    Technologies in Sustaining K-12 Schools during COVID-19. ICST Transactions on Mobile Communications and Applications, 168509. https://doi.org/10.4108/eai.29-1-2021.168509

Hwang, S.-Y., & Kim, J.-N. (2021). A Malware Distribution Simulator for the Verification of Network Threat Prevention Tools. Sensors, 21(21), 6983. https://doi.org/10.3390/s21216983

Jadhav, S., Thakur, A., Nalbalwar, S. L., Shah, S., & Sankalp Chordia. (2023). Detection and Mitigation of ARP Spoofing Attack. Lecture Notes in Networks and Systems, 383–396. https://doi.org/10.1007/978-981-99-3010-4_33

Kara, İ. (2024). Twin Ghosts: Evil Twin Attacks in Wireless Networks and Defense Mechanisms. Bitlis Eren University Journal of Science and Technology. https://doi.org/10.17678/beuscitech.1450756

Klein, J. D., & Walcott, K. R. (2021). Exploiting Telnet Security Flaws in the Internet of Things. https://doi.org/10.1007/978-3-030-12385-7_51

Lakhani, R., & Sachan, R. C. (2024). Securing Wireless Networks Against Emerging Threats: An Overview of Protocols and Solutions. Journal of Science & Technology, 5(4), 132–158. https://doi.org/10.55662/jst.2024.5406

Li, J., Xiao, W., & Zhang, C. (2023). Data security crisis in universities: identification of key factors affecting data breach incidents. Humanities & Social Sciences Communications, 10(1), 270. https://doi.org/10.1057/s41599-023-01757-0

Lim, Y. Z., Abdul, & Sikdar, B. (2025). False Sense of Security on Protected Wi-Fi Networks. ArXiv.org. https://arxiv.org/abs/2501.13363

Morsy, S. M., & Nashat, D. (2022). D-ARP: An Efficient Scheme to Detect and Prevent ARP Spoofing. IEEE Access, 10, 49142–49153. https://doi.org/10.1109/access.2022.3172329

Ntwari, R., E. Habinka, A., & Kaggwa, F. (2021). Enhancing Bring Your Own Device Security in Education. Journal of Science & Technology, 02(04), 69–86. https://doi.org/10.55662/jst.2021.2401

OWASP IoT Security Testing Guide - OWASP IoT Security Testing Guide. (n.d.). Owasp.org. https://owasp.org/owasp-istg/

Palamà, I., Amici, A., Bellicini, G., Gringoli, F., Pedretti, F., & Bianchi, G. (2023). Attacks and vulnerabilities of Wi-Fi Enterprise networks: User security awareness assessment through credential stealing attack experiments. Computer Communications, 212, 129–140. https://doi.org/10.1016/j.comcom.2023.09.031

Philipp Pütz, Mitev, R., Miettinen, M., & Sadeghi, A.-R. (2023). Unleashing IoT Security: Assessing the Effectiveness of Best Practices in Protecting Against Threats. Annual Computer Security Applications Conference. https://doi.org/10.1145/3627106.3627133

Polin, K., Yigitcanlar, T., Limb, M., & Washington, T. (2023). The Making of Smart Campus: A Review and Conceptual Framework. Buildings, 13(4), 891. https://doi.org/10.3390/buildings13040891

Ulven, J. B., & Wangen, G. (2021). A Systematic Review of Cybersecurity Risks in Higher Education. Future Internet, 13(2), 39. MDPI. https://doi.org/10.3390/fi13020039

Vijay Bhuse, Yesaswini Vellaboina, & Wang, X. (2024). Enhancing Network Security: Rogue Switch Detection and Prevention in Local Area Network. European Conference on Cyber Warfare and Security, 23(1), 66–73. https://doi.org/10.34190/eccws.23.1.2282

What is a Rogue Access Point Attack? (n.d.). Portnox. https://www.portnox.com/cybersecurity-101/rogue-access-point-attack/

Wireless Security Architecture. (2022). Google Books. https://books.google.com.my/books?id=BzVjEAAAQBAJ&dq=securing+enterprise+wireless+networks&lr=&source=gbs_navlinks_s

Yuhong, Y., Zhuo, S., & Monreal, R. N. (2023). Design of the Network Security Architecture for Smart Campus in the Philippines. Journal of Knowledge Learning and Science Technology ISSN: 2959-6386 (Online), 2(1), 26–34. https://doi.org/10.60087/mrb0hh55

Zaidi, S. A. N. (2024). Mitigating DDoS Attacks on IoT Networks: Strategies and Solutions. International Journal for Research in Applied Science and Engineering Technology, 12(6), 1387–1394. https://doi.org/10.22214/ijraset.2024.63246